| [ INSERT YOUR COMPANY NAME HERE ] Maintenance Standard Operating Procedure | Created By: ____________________ Verified By: ____________________ Version: 1.0 · Est. Time: 45 min |
| --- | --- |

Edgebander Trim Saw Dust Hood Seal Integrity Inspection & Replacement

1.0 Maintenance Objective & Frequency

The objective of this procedure is to inspect and replace the dust hood seals on the edgebander trim saw to prevent dust leakage, reduce contamination of bearings and guide rails, and maintain proper dust extraction efficiency. A compromised seal can lead to airborne particulate hazards, increased wear on moving components, and potential fire risk from accumulated dust.

Frequency: Every 500 operating hours or monthly, whichever comes first. More frequent inspection is required if the machine operates in high-dust environments or processes MDF/particleboard.

2.0 Lockout/Tagout (LOTO) & Safety

• Electrical Isolation: Disconnect main power to the edgebander at the disconnect switch. Apply personal lock and tag. • Pneumatic Isolation: Close the compressed air supply valve to the machine. Bleed residual air from the dust hood cylinder by actuating the manual override. • Mechanical Lockout: Ensure the trim saw blade has come to a complete stop before accessing the dust hood. • Mandatory PPE: Safety glasses, cut-resistant gloves, hearing protection, and a NIOSH-approved N95 respirator (due to fine wood dust).

3.0 Required Tools, Parts & Lubricants

▪  Replacement dust hood seal kit (OEM part number per machine manual)

▪  5mm hex key (Allen key)

▪  8mm hex key (Allen key)

▪  Flathead screwdriver (medium)

▪  Torque wrench (Nm range 10-50)

▪  Digital caliper (for seal gap measurement)

▪  Shop vacuum with HEPA filter

▪  Clean lint-free rags

▪  Isopropyl alcohol (99%) for cleaning seal surfaces

▪  Silicone-free lubricant (e.g., Super Lube 21030)

4.0 Step-by-Step Maintenance Procedure

1.  Perform full LOTO as per Section 2.0. Verify zero energy state by attempting to start the trim saw (it should not run).

2.  Remove the dust hood access panel using a 5mm hex key. Set aside the panel and fasteners in a clean location.

3.  Inspect the dust hood seal for visible wear, cracks, compression set, or delamination from the hood. Use a flashlight to examine the full perimeter.

4.  Measure the seal gap between the hood and the saw arbor housing using a digital caliper. Record the measurement. Acceptable gap is 0.5 mm to 1.5 mm. If gap exceeds 2.0 mm, replacement is required.

5.  If seal replacement is required, remove the retaining screws using an 8mm hex key. Carefully pry the old seal off using a flathead screwdriver, taking care not to damage the hood surface.

6.  Clean the seal mounting surface thoroughly with isopropyl alcohol and a lint-free rag. Remove all adhesive residue and dust. Allow to dry completely.

7.  Install the new seal by pressing it firmly into the groove. Ensure it is seated evenly without twists. Apply a thin bead of silicone-free lubricant to the seal lip to reduce friction during operation.

8.  Reinstall the retaining screws and torque to 12 Nm using a torque wrench. Do not overtighten as this can distort the seal.

9.  Reinstall the dust hood access panel and torque the fasteners to 8 Nm.

10.  Remove LOTO devices and restore power and compressed air.

5.0 Verification & Return to Service

1.  Activate the dust extraction system and run the edgebander through a test cycle on a scrap workpiece. Observe the dust hood for any visible dust leakage at the seal interface.

2.  Use a smoke pencil or thermal anemometer to check for air leaks around the seal. No visible smoke should escape.

3.  Verify that the dust hood opens and closes smoothly without binding. Listen for any abnormal rubbing sounds.

4.  Measure the seal gap again with a digital caliper after the test cycle. Confirm it remains within 0.5 mm to 1.5 mm.

5.  Document the inspection results, seal gap measurements, and any parts replaced in the PM work order within ServiceGrid. Sign off and close the work order.

Regulatory Disclaimer: This document is a generic maintenance template provided for educational and informational purposes only. It does not constitute professional engineering or reliability advice. The end-user assumes all liability for validating all step sequences, torque specs, and LOTO protocols against specific machinery OEM manuals before deployment to a live production floor.